Master Project: BUNO

Professor

Ayca Tuzmen

- Team Number & Team Members

Group 3

Tran Cong Minh Nguyen

Abdullah Burak Karatepe

Aaron Varghese James

- Product Description

Most of us have played UNO at least once. It's loud, it's fun, and someone always gets angry when a Draw Four gets thrown down. But for someone who can't see, even picking up a card and knowing what's in their hand is a challenge. That one small thing most of us never think about makes the whole game out of reach.

That's what BUNO is trying to fix.

It started as a simple idea: what if a blind person could just press a button, hold up a card, and be told what it is? No special deck, no modifications to the game, no needing someone else to read it out for them. Just play the game like everyone else.

So we built it. A small 3D-printed frame sits on the table. There's a tiny camera inside that looks down at the card. When you press the button, it takes a photo, figures out the color and number, and within about a second, it speaks through an earphone, "Blue 7", "Green 4", whatever the card is. A small buzz confirms it worked, kind of like your phone vibrating when a message comes in.

The whole thing runs on a Raspberry Pi, a mini computer about the size of your palm tucked inside a little plastic box under the frame. It's not expensive. It's not complicated to use. You press one button.

What we liked most about building this was realizing how little it takes to make something genuinely useful. It's not a big invention. It's just paying attention to a problem that most people walk past without noticing and deciding to do something about it.

Because everyone deserves to slam down a Wild card and enjoy the chaos that follows.

- Product Photos

- State Diagram

- Circuit Design in TinkerCAD

- Gantt Chart

The project BUNO started on March 17, 2026 and was completed on April 16, 2026. Throughout this period, tasks were planned and executed according to the Gantt Chart.

- Week 1 (Mar 17–19): We focused on initial planning — defining gameplay logic, designing the system flow, researching components, and creating the first circuit layout.

- Week 2 (Mar 24–26): We moved into development — building the physical design, testing key inputs (camera and button), and implementing core features such as mode switching, Bluetooth connection, and AI-based color recognition.

- Week 3 (Mar 31 – Apr 2): We worked on integration — completing hardware connections, implementing system behavior, and finalizing code and documentation.

- Week 4 (Apr 7–9): We conducted testing and refinement — improving hardware durability, running system tests, fixing issues, and collecting user feedback.

- Week 5 (Apr 14–16): We finalized the project — summarizing feedback, evaluating team collaboration, and preparing improvement and implementation plans.

- Powerpoint Presentation

PROJECT POWERPOINT.pptx

- Product Functionality Video

https://youtu.be/gJ1cViXSRvw

- Members Contribution Video

https://youtu.be/Ka0sSeJKYyM

- Code

#!/usr/bin/env python3

import RPi.GPIO as GPIO

import subprocess

import base64

import sys

import os

import time

from openai import OpenAI

# --- Configuration ---

CAPTURE_BTN_PIN = 11

MOTOR_PIN       = 13

client = OpenAI(api_key="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")

MODEL = "gpt-5-nano-2025-08-07"

IMAGE_PATH = "/home/pi/captured_image.jpg"

def setup():

GPIO.setmode(GPIO.BOARD)

GPIO.setwarnings(False)

# Motor OFF (Low)

GPIO.setup(MOTOR_PIN, GPIO.OUT)

GPIO.output(MOTOR_PIN, GPIO.LOW)

# Button: Normal=1, Pressed=0

GPIO.setup(CAPTURE_BTN_PIN, GPIO.IN, pull_up_down=GPIO.PUD_UP)

print("System ready. Waiting for button...")

def haptic_feedback(duration):

GPIO.output(MOTOR_PIN, GPIO.HIGH) # ON

time.sleep(duration)

GPIO.output(MOTOR_PIN, GPIO.LOW)  # OFF

def speak(text):

"""Voice output for the earphones"""

print(f"VOICE: {text}")

# This command sends the text to your Bluetooth audio

os.system(f'espeak "{text}" 2>/dev/null')

def extract_text(resp):

text = getattr(resp, "output_text", None)

if text:

return text.strip()

try:

for item in getattr(resp, "output", []):

for part in getattr(item, "content", []):

if getattr(part, "type", None) in ("output_text", "text") and getattr(part, "text", None):

return part.text.strip()

except Exception:

pass

return str(resp)

def run_uno_scan():

print("\n--- Scan Triggered! ---")

haptic_feedback(0.2)

try:

# Capture image

subprocess.run(["fswebcam", "-r", "640x480", "-S", "2", "--no-banner", IMAGE_PATH], check=True)

# Encode

with open(IMAGE_PATH, "rb") as f:

data_url = f"data:image/jpeg;base64,{base64.b64encode(f.read()).decode('utf-8')}"

# The Prompt

prompt = "Identify the UNO card color and number. Reply with just the color and number, e.g., 'Red 8'."

resp = client.responses.create(

model=MODEL,

reasoning={"effort": "low"},

input=[{"role": "user", "content": [

{"type": "input_text", "text": prompt},

{"type": "input_image", "image_url": data_url}

]}]

)

result = extract_text(resp)

print(f"RESULT: {result}")

# --- NEW: Speak the result out loud ---

speak(result)

# Success signal: Two quick buzzes

for _ in range(2):

haptic_feedback(0.1)

time.sleep(0.1)

except Exception as e:

print(f"Error: {e}")

speak("Error scanning card")

def main():

setup()

while True:

if GPIO.input(CAPTURE_BTN_PIN) == GPIO.LOW:

run_uno_scan()

while GPIO.input(CAPTURE_BTN_PIN) == GPIO.LOW:

time.sleep(0.1)

time.sleep(0.1)

if __name__ == "__main__":

try:

main()

except KeyboardInterrupt:

GPIO.cleanup()

- Relevant Documentations

- 3D Print Design

- Photo Captured